Spatial Measurement of Bedload Transport in Tigris River

Ammar A. Ali1 , Nadhir A. Al-Ansari2*, Qusay Al-Suhail3, Sven Knutsson2

1Water Resources, College of Engineering, University of Baghdad, 10071, Baghdad, Iraq

2 Department of Civil, Environmental and Natural Resources Engineering, Lulea University of Technology, 97187, Lulea, Sweden

3 Earth Sciences, College of Science, University of Baghdad, 10071, Baghdad, Iraq

* Corresponding author

Emails: ammali_75@yahoo.com , nadhir.alansari@ltu.se , sven.knutsson@ltu.se ,  quab65@hotmail.com

Abstract Using Helley-Smith sampler, 288 bedload samples were collected from 16 cross sections along 18 km reach length of Tigris River within Baghdad. The spatial distribution of sampling along the reach took into consideration the variance of river topography where 7 meanders, 2 islands and several bank depositions characterize the geometry of the river. The implemented regulation schemes on Tigris River have reduced 44% of water discharges compared to previous period. The spatial variance in topography was effectively scattering the results of the applied twenty bed load formulas. The study results indicated that the complicated geometry of the river reach makes finding a unique representative bedload formula along the study reach rather difficult, and there is no grantee to have good agreement with measurements in the irregular cross sections (meanders, sand bars, etc.). The closest bedload prediction formulas were van Rijn1984. The annual transported quantities of bedload were estimated to be 30 thousand tons (minimum) in 2009 and 50 thousand tons (maximum) in 2013.

Keywords: bedload sampling, spatial bedload, Helley-Smith sampler, meandering river, sand bed, prediction formula, Tigris River.

- Introduction

Natural rivers transport sediment in two modes, suspended load and bedload. The former is finer and it is transported in suspension whether its source is wash load or bed sediment. While the latter “bedload” is the coarser fraction and it moves in contact with the river’s bed by sliding, rolling and saltating according to the boundary shear stress. An exchange occurs between suspended and bed loads, and between bedload and bed material depending on the sizes of sediment particles, transport capacity, flow velocity, and boundary shear stress (Hickin 1995, WMO 2003).

The main sources of fluvial sediments are watershed erosion, stream erosion and human activities (Vanoni 2006). Since sediment sources are unlimited and streams have sufficient sediment transport capacity, sediment transport will continue (Friend 1993). Limited Supply condition of the fine sediment is the case in most natural rivers. When the river transport capacity reduces, certain range of sediment sizes becomes heavy to be kept transported even as bedload, then the river becomes competence-limited case (Hickin 1995). River bed material is the source of one of the sediment load components which is called bed-sediment load. This load’s component is transported as bed load or even in suspension and it should be distinct from the wash load as sediment source (Hickin 1995, WMO 2003).

Prediction of bedload is of primary importance for river engineering and geomorphology (Recking 2009). Its effect on developing the bed forms, driving fluvial incision and knick point propagation (Cook et al. 2013),

It is difficult to measure bedload directly because the measuring sampler performance is affected by several parameters such as hydraulic efficiency, sampler orientation, bed-forms, bed material and so on (Gaudet et al. 1994; Hickin 1995). However, a lot of measurements were conducted in labs and natural streams (Helley and Smith 1971, Gaweesh and Van Rijn 1994, Bunte and Abt 2009, Atkinson 1994; Strahlhofer 2010) using different samplers, such as manually operated portable samplers, vortex tube, pit and trough (Diplas et al. 2008).

In this work, an attempt has been made to calculate the bedload transport rate of the northern part of Tigris River within Baghdad directly using field measurement and to predict sediment bed load indirectly using mathematical formulas.

- Tigris River

Tigris River bisects Baghdad City, the capital of Iraq, in two parts (Fig. 1) for a distance of 50 km within urban zone and 10 km within rural zones (Ali 2013, Ali et al. 2012, 2014). The northern part of Tigris River reach, which is considered in this work, of 18 km length extends from Al-Muthana Bridge to the north to Sarai Baghdad gauging station at the center of Baghdad (Fig. 1.B). This river reach has single thread, compound meanders, and alluvial plain characteristics. The river banks are protected against erosion by aligned stones and cement mortar between levels 29 and 37 m.a.s.l. Recently, the dominant water levels in the reach are below the protection levels (Ali 2013, Ali et al. 2012, 2014).

During the last two decades many new islands, side depositions and point bars appeared in the Tigris River’s reach within Baghdad (Fig. 1.B). These sedimentations in the river course has its impact on the hydraulic performance of the river, such as reducing its flood capacity, impeding navigation and reducing the efficiency of water intakes of water treatment plants, as well as the environmental and aesthetic impacts (Ali et al. 2012, 2014).

Previous study about sediment transport and river training (Geohydraulique 1977) which was conducted on the river reach in Baghdad, mentioned Tigris River sediment is “bed-load”. Since it found the suspended load concentrations never reach 3 g l-1 in high water and never exceed 0.2 g l1 in low water periods. This study was not based on real field measurements for the bedload. Bed material samples and suspended load samples were collected only in that study. Therefore, it is important to measure the bedload discharge in Baghdad since no measurements were previously performed.

Euphrates River

Tigris River

Shatt Al-Arab

Arabian Gulf

Baghdad City

Sarai Baghdad Gauging Station

Al-Muthana Bridge

Diyala River

Northern

Part

Southern

Part

A

B

Figure 1: (A) Map of Iraqi Provences (B) Tigris River inside Baghdad, capital of Iraq (islands and sandbars bordered by red) (Ali et al. 2014, 2015)

  - Hydrology of Tigris River in Iraq

The flow of the river is fully controlled in Baghdad by a system of dams and regulators constructed on the main river and the tributaries upstream of Baghdad (Fig. 2)(Al-Ansari 2013& 2016). These regulating schemes have decreased the average monthly discharge of the river 44% according to the records of the Sarai Baghdad gauging station, it has been 522 m3s-1. The Tigris River hydrograph at Sarai Baghdad (Fig. 3) shows the main delivery events of sediment into Baghdad have been vanished.

Sediment transport rates are affected in the course of Tigris River upstream of Baghdad due to the trapping of sediment within the reservoirs of the headwater.

The only uncontrolled source of sediment that can be delivered to Baghdad is the area restricted from the lower sub-basin of the Adhaim tributary and the catchment between the Samarra Barrage and Baghdad (Fig. 2), as well as the bed and banks erosions.

The delivery of fine sediment from the Adhaim Tributary has not been measured, but a glance at the possible extra flow contribution (rather than flow released from the Adhaim Dam), can give an indication for the estimated sediment delivery. The extra water flow contribution from the Adhaim Tributary sub-basin and Tharthar Lake back feed toward the Tigris River was determined using the mass balance concept. The contribution did not exceed 260 m3s-1 during 2004-2005, which was a moderate year compared with recent more dry years as shown in figure (4). As an average, the extra contribution was 8% of the average monthly discharge at the Sarai Baghdad for the same year.

Baghdad

Mosul

Iraq–Turkey Border

Mosul

Dokan

Derbendi

Khan

Hemrin

Al-Adhaim

Khabour

Greater Zab

Lesser Zab

Al Khazir

Diyala Weir

Diyala  River

Samarra

Adhaim Tributary

Flood Escape Channel

Tharthar-Tigris Canal

Tharthar

Lake

Tigris  River

Fatha/Baji

Duhok

Iraq –Iran  Border

Dibs

Tigris  River

To Kut

Iraq –Iran  Border

City

Dam

Barrage

Regulator

Natural River

Irrigation Canal

Possible source of extra sediment

Figure 2: Schematic Diagram of Tigris River Hydrological Scheme (MWR 2005).

Figure 3: Hydrographs of Tigris River at Sarai Baghdad for the period 1930-2013 (data source: Al-Shahrabaly 2008).

Figure 4: Contribution of runoff of Adhaim River sub-basin downstream Adhaim Dam to Tigris River discharges for the year 2004~2005.

  - River Geometry and Bed Composite

The morphology and the bed sediment of the river were investigated several times inside Baghdad by Geohydraulique (1977), Al-Ansari and Toma (1984), Khalaf (1988), University of Technology (1992) and Al-Ansari et al. (2015). Rapid changes in the bed material from gravel in Samarra to fine sand in Baled (50 km to the downstream of the Samarra Barrage and 130 km to the upstream of Baghdad) (Hardy et al. 1956). The riverbed was already sand bed between Baled and Baghdad even before the construction of the Samarra Barrage in 1956.

The geometry of the study area consists of a series of 7 meanders (Fig. 5) of radii of curvature are ranging from 475m to 1245m (University of Technology 1992). Along the second meander (CS4, CS4-2 and CS5) an island is noticed directly upstream of CS4 and the river cross section transfers from riffle at CS4 to pool at CS4-2 and CS5 and the higher velocity zone is also transferred from the inner bank of CS4 to the center of CS4-2 then to the outer bank of CS5. This change in the velocity field gives an indication about the attempts of the river to shift the peak of the meander to the downstream of its’ current location. This can explain the high depositions on the outer bank between CS4 and CS4-2.

Al-Muthana Bridge

Sarai Baghdad

Figure 5: Bedload sampling cross sections along the northern part of Tigris River (Ali et al. 2014, 2015)

The same velocities distribution is repeated at the 4th and 5th meanders (CS8 and CS10 respectively), where the higher velocities are in the centers of the sections and large depositions existed in the front halves of the concave banks.

At the third meander (CS6-1, CS6-2, CS6-3 and CS6-4), a large island mediates the meander. The sections CS6-1 and CS6-4 are of pool type while the sections CS6-2 and CS6-3 are of run type. Depositions are existed along the inner bank of the meander. Mean velocities in both branches are equal while the top width of the right branch is 2.2 times the left branch.

Using the van Veen grab, 46 bed-material samples were collected along the northern reach in Baghdad. The particle size distribution was analysed using the sieves and the hydrometer. It was noticed that fine sand dominant the riverbed. The average median size was 0.178 mm (Al-Ansari et al. 2015b). The size of the bed sediment relatively decreased compared to earlier investigations (see Al-Ansari and Toma 1984). In addition, the sediments were moderately sorted, fine skewed and leptokurtic (Al-Ansari et al. 2015b).

- Bedload Sampling

A Helley-Smith sampler was manufactured of 3" × 3" (76 mm × 76 mm) opening size and 3.5 exit/entrance expansion ratio. Other dimensions of the sampler were taken from van Rijn (2007). Two exceptions were considered, the weight of the sampler and the size of the mesh bag. The original weight of the sampler was reduced to 15kg, which makes it easier handling in small boat without winch. No problem in the performance was expected to be due to the reduced weight since the average velocity in the river did not reach 1 m s-1. Furthermore, avoiding the oversampling that may occur due to the scooping effect (Gaweesh and Van Rijn 1994, Bunte and Abt 2009) is more valuable. Larger mesh bag of 2200 cm2 surface area was used to maintain the sampling efficiency in case the bag is filled for more than 40% of volume and to avoid clogging the bag openings by close size particles and organic materials (Druffel et al. 1976, Beschta 1981, Van Rijn and Gaweesh 1992).

At the quartiles of 16 cross sections (Fig. 5), 288 bedload samples were collected. Sampling times were 60s of 238 samples, 120s of 2 samples, and 300 s of a single sample. The separation period was 3 min between 5 sequences samples at each sampling point to determine the time-averaged rate of sediment transport. Zero-time samples were collected at the sampling points to overcome the initial and scooping effect of the sampler on the bed (Van Rijn and Gaweesh 1992). Velocity profile measurements were conjugated with bedload sampling in all cross sections using SonTek RiverSurveyor ADCP.

The mean weight was determined for the repetitions of the same sampling point. The weights of zero-time samples were subtracted from the means. A reduction factor of 0.5 was applied to the modified means considering the trapping efficiency as 200% for fine sand. Table (1) shows the measured bedload discharge for each cross section along the reach with the bed sediment properties and the hydraulic-geometric parameters those are associated or existed during measurements.

The maximum bedload transport rate was 3.938 kg s-1 at CS8 associated with a water discharge of 643.5 m3s-1 and the minimum was 0.6822 kg s-1 at CS4 associated with a water discharge of 449 m3s-1. The average bedload transport rate was 2.099 kg s-1 with 0.889 standard deviation.

The scattering in some of bedload measurements can be attributed to the spatial variation in river topography along the study reach where it is an influencing factor, as well as, bed sediment size, particle size distribution and bed shear stresses (Julien 2010) in case there is no external source of disturbance.

- Spatial Distribution of Bedload

The variance in the topography and morphology in the Tigris River was reflected in the spatial distribution of bedload and velocity field as shown in figure (6). This figure shows the distribution of the measured bedload discharges per unit width at sampling points along the study reach as well as bed shear stresses. The data series are not in the sequence of display in the figure and separation lines were used to specify cutting in the data series and also to specify the relative parts of each data series to a certain cross section. The following description for the spatial distribution of bedload was associated with the measured velocity distribution using ADCP at the sampling time.

Table 1 The measured bedload rates with the bed sediment properties and hydraulic-geometric parameters along the northern reach of Tigris River.

| C.S. | d50 (mm) | d90 (mm) | Cross-sectional Area (m2) | Top Width (m) | Hydraulic Radius (m) | Discharge (m3s-1) | Water velocity (ms-1) | Bedload rate (kgs-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS1 | 0.194 | 0.273 | 664.9 | 180.03 | 2.98 | 457.381 | 0.688 | 1.268 |
| CS2 | 0.166 | 0.235 | 653.7 | 260.77 | 2.471 | 459.022 | 0.702 | 2.590 |
| CS3 | 0.1755 | 0.25 | 795.2 | 261.6 | 3.008 | 464.409 | 0.584 | 1.341 |
| CS4 | 0.199 | 0.273 | 691.5 | 250.06 | 2.743 | 445.095 | 0.644 | 0.682 |
| CS4-2 | 0.197 | 0.276 | 745 | 241.09 | 3.013 | 452.325 | 0.607 | 1.142 |
| CS5 | 0.208 | 0.278 | 643.3 | 151.67 | 4.072 | 489.233 | 0.76 | 1.599 |
| CS6-1 | 0.199 | 0.273 | 865.2 | 353.85 | 2.398 | 549.877 | 0.636 | 2.901 |
| CS6-2 | 0.21 | 0.275 | 421.284 | 185.2 | 2.271 | 286.409 | 0.68 | 0.924 |
| CS6-3 | 0.145 | 0.255 | 369.83 | 83.37 | 4.113 | 251.023 | 0.679 | 0.190 |
| CS6-4 | 0.19 | 0.27 | 760.4 | 237.8 | 3.141 | 561.778 | 0.739 | 3.420 |
| CS7 | 0.2 | 0.277 | 932.7 | 320.08 | 2.881 | 651.709 | 0.699 | 1.564 |
| CS8 | 0.2 | 0.275 | 979 | 236.86 | 4.053 | 643.319 | 0.657 | 3.938 |
| CS9 | 0.12 | 0.218 | 772.9 | 255.5 | 2.911 | 530.443 | 0.686 | 2.171 |
| CS11 | 0.143 | 0.243 | 1128.40 | 213.99 | 5.279 | 578.375 | 0.513 | 2.716 |
| CS13 | 0.197 | 0.276 | 720.6 | 114.9 | 5.889 | 529.965 | 0.735 | 2.630 |
| CS14 | 0.135 | 0.213 | 711.4 | 137.84 | 4.976 | 522.226 | 0.734 | 2.579 |

Figure 6: The measurements of the bedload discharges with the measured velocity and the calculated bed shear stresses at the sampling points along the Tigris River study reach.

CS1: 	The section is a run section type. Some stagnation was on the extreme left side because it is located in the shade of bank deposition. Bedload was oscillating across the section. Velocity was uniform except on the extreme left side

CS2: 	Bank deposition was growing on the left side where flow velocity was low. Bedload was higher towards the right.

CS3: 	The section is located directly downstream of a large bank deposition. Its left side is hidden by the deposition and the velocity was higher on the right side. The bedload was low in the section, however it was little higher on the right side.

CS4: 	The section is located between a small island to the upstream and an acute meander to the downstream. The right side was stagnant because it is hidden behind the island and the velocity increased towards the left side. The bedload was higher towards the left side. Eddies were noticed on the right side.

CS4-2: 	The section is pool type, deeper on the right side. It’s located at the center of an acute meander. On the deeper side, velocity was not at its highest, higher velocities were closer to the center. The bedload was higher in the center.

CS5: 	The section is pool type, deeper on the right side. Velocity and bed shear were higher on the right side. However, the bedload was higher in the center of the section due to the effect of the secondary flow, since the section is within the downstream half of an acute meander.

CS6-1: 	The section is pool type; deeper on the left side. The velocity distributed uniformly across the section. The bedload was higher on the left side.

CS6-2: 	The section is the inner branch of a meander that is bisected by an island. The left side of the section was a trench produced by excavators, so the velocity was low by comparison with the right side. The bedload was higher on the right side where the velocity was higher.

CS6-3: 	The section is the outer branch of the meander. The flow was turbulent on the most outer side. Bedload was low at all.

CS6-4: 	The center of the section was close to the tail of a large island, so the secondary flow at the confluence of the two branches was the reason behind the high bedload on both sides.

CS7: 	The section is of a riffle type. The velocity on the left side was higher, while on the right side, although the bed shear was relatively high, the bedload was low because the velocity was also low.

CS8: 	The section is pool type; deeper on the right side. However, the velocity was neither high nor was the bedload. The highest bedload was in the center due to the effect of the secondary flow.

CS9: 	The main flow was on the right side while the left side was stagnant, so bedload was higher on the right and lower on the left.

CS10: 	The section is pool type, deeper on the left side. The bedload was lower due to higher bed shear, which may suspend the bed sediment.

CS11: 	The section is the riffle type. Although the right side has the same depth as the left, but bedload was much higher on the left because the right side was stagnant and recently dredged.

CS13: 	The section is pool type, deeper on the right. The bedload was lower on the right due to higher bed shear, which may suspend the bed sediment. The bedload was higher in the center due to the effect of secondary flow.

CS14: 	The section is pool type, deeper on the left. The bedload was lower on the left due to higher bed shear and higher flow velocity, suspend the bed sediment.

- Bedload Prediction Using Formulas

Wide spectrums of bedload predicting formulas were proposed and developed by many researchers depending on different approaches. For each approach, a specified concept was considered as motivation for deriving the approach’s formula and a certain number of parameters were controlled in the lab measurements to estimate the formula parameters.

  - Approaches of Bedload Formulas

Twenty bedload formulas were selected and applied on the study reach to predict the bedload discharge to find the best suitable formulas. Brief descriptions for the used approaches are given below:

    - Shear stress approach

The movement of bed material particles will start when the criteria of incipient motion is exceeded. So, shear stress near the bed will entrain the sediment particles to motion as long as the shear stress is greater than the critical shear stress of the particles. The following formulas which belong to this approach were used in this work:

- DuBoys1935 formula (Graf 1971)

- Shields1936 formula (Yang 1996)

- Kalinske1947 formula (Yang 1996)

- Cheng-Simons-Richardson1965 formula (Yang 1996)

- Wong-Parker2006 formula (Wong and Parker 2006)

    - Energy slope approach

The bedload motion is initiated due to the portion of energy losses coming from the grain resistance (Yang 1996). The following formulas were used from this approach:

- Meyer-Peter1934 formula (Vanoni 2006)

- Meyer-Peter-Muller1948 formula (Graf 1971)

    - Discharge approach

In natural rivers, critical unit discharge was used as an indication on starting bedload sediment motion when it is exceeded by water discharge (Talukdar et al. 2012). The following formulas which belong to this approach were used in this work:

- Schoklisch1934, 1943 formula (Yang 1996)

- Casey1935 formula (Khalaf 1988)

    - Probabilistic approach

Probability concepts were introduced in bedload prediction by the pioneer work of Einstein in 1942. The turbulent flow fluctuations are the driver for sediment entrainment rather than the flow forces exerted on the particle. Both of the entrainment and the deposition were expressed in probability terms (Yang 1996).

- Einstein1950 bedload function

- Einstein-Brown1950 formula

    - Regression approach

Data driven models (regression, ANN) were used to explain the bedload transport process due to the limitations of defining this complex process into precise formula (Talukdar et al. 2012). The following formulas were used within this approach:

- Rottner1959 formula

- Yalin1963 formula (Yalin 1977)

- Van Rijn1984 formula (van Rijn 1993)

- Julien2002 formula (Julien 2002)

- Camenen-Larson2005 formula (Camenen and Larson 2005)

    - Equal mobility approach

The flow forces act on the exposed particles causing mobilization with possibility of participat0in of the substrate particles into bedload movement at scour zones due to their exposure on the surface (Yang 1996).

- Wilcock2001 formula (Wilcock 2001)

- Wilcock-Crowe2003 formula (Wilcock and Crowe 2003)

    - Power Concept

This approach has developed from the concept that there is a relation between the available energy to the river with the rate of work done by the river to transport sediment (Yang 1996). The following formula was used within this approach:

- Bagnold1966 formula

  - Application of Bedload Formulas

Two kinds of datasets were required for applying bedload formulas, physical properties of river bed sediment and hydraulic-geometric parameters of the study reach. Sediment characteristics were determined from the size analysis of the bed materials samples. The hydraulic-geometric parameters included; water depth, cross sectional area, top width, wetted perimeter, hydraulic radius, water surface slope and water discharge. These datasets were extracted from field measurements in the sampled cross sections. The results published by Al-Ansari et al. (2015b) contained most of the datasets, whilst other datasets were listed in table 1.

The results of the bedload formulas at sixteen cross sections along the study reach were compared with the measured bedload discharges in the same section and two indicators were used to measure the accuracy of the predicted bedload. The discrepancy ratio, which is the ratio of predicted bedload to measured one (van Rijn 1993), was one of the indicators and the error percentage (Walling 1977) was the other. The comparisons of results are shown in figure (7). Six zones of different discrepancy ratios were specified in the figure to explain the distribution of the results around the perfect agreement line.

Most of the formulas overestimated the bedload transport rate by more than 10 times and even 100 times relative to field measurements. Five formulas from four of the approaches predicted bedload discharges close to measurements. These formulas were Meyer-Peter1934, Schoklitsch1934, 1943, van Rijn1984 and Einstein1950 bedload function with average discrepancy ratios of 0.5, 1.51, 0.47, 1.18 and 4.06 respectively. The predictions of van Rijn1984 and Schoklitsch1934 formulas are distributed on both sides of the perfect agreement line. Whilst both of Meyer-Peter1934 and Schoklitsch1943 formulas are mainly bounded between the perfect line and discrepancy ratio ¼. Some results of Einstein1950 were in the area between the perfect agreement and r = 8.

Table (2) shows the accumulated percentages of the predicted bedload discharges according to each range of the discrepancy ratio. The higher percentage of predicted bedload within the closer range of discrepancy ratio 0.75 ~ 1.25 (Error%= -25 ~ +25) was equally between Schoklitsch1934 and van Rijn1984 formulas and the results approximately continued in this manner until the third zone of discrepancy ½ ~ 2 (Error%= -50 ~ +100). At this range, more than 76% of Schoklitsch1934 and 53% and of van Rijn1984 predictions were located within the range. The percentages of the other three formulas didn’t exceed 24% for the discrepancy range ½ ~ 2 (Error%= -50 ~ +100).

Figure 7: Comparison of bedload discharges predicted by different bedload formulas with measured bedload in Tigris River.

Table 2 Accumulative percentages of predicted bedload according to the ranges of discrepancy ratio.

| Formulas | Ranges of discrepancy ratio and corresponding error percentages |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0.75 ~ 1.25 | 0.67 ~ 1.5 | ½ ~ 2 | ¼ ~ 4 | ⅛ ~ 8 |
|  | -25 ~ +25 | -33 ~ +50 | -50 ~ +100 | -75 ~ +300 | -87.5 ~ +700 |
| Meyer-Peter 1934 | 5.88 | 5.88 | 17.65 | 76.47 | 100 |
| Schoklitsch 1934 | 23.53 | 47.06 | 76.47 | 82.35 | 88.24 |
| DuBoy 1935 | 0 | 0 | 0 | 0 | 0 |
| Casey 1935 | 0 | 0 | 0 | 0 | 0 |
| Shield 1936 | 0 | 0 | 0 | 0 | 0 |
| Schoklitsch 1943 | 5.88 | 5.88 | 23.53 | 82.35 | 100 |
| Kalinske 1947 | 0 | 0 | 0 | 0 | 0 |
| M-P-M 1948 | 0 | 0 | 0 | 0 | 11.76 |
| Einstein 1950 | 5.88 | 11.76 | 23.53 | 41.18 | 64.71 |
| Enistein-Brown 1950 | 0 | 0 | 0 | 0 | 23.53 |
| Rottner 1959 | 0 | 0 | 0 | 5.88 | 23.53 |
| Yalin 1963 | 0 | 0 | 0 | 0 | 0 |
| Chang et al. 1965 | 0 | 0 | 0 | 23.53 | 70.59 |
| Bagnold 1966 | 0 | 0 | 0 | 0 | 0 |
| van Rijn 1984 | 23.53 | 41.18 | 52.94 | 94.12 | 94.12 |
| Wilcock 2001 | 0 | 0 | 0 | 17.65 | 47.06 |
| Julien 2002 | 0 | 0 | 0 | 0 | 0 |
| Wilcock-Crowe 2003 | 0 | 0 | 0 | 0 | 29.41 |
| Camenen-Larson 2005 | 0 | 0 | 0 | 0 | 0 |
| Wong-Parker 2006 | 0 | 0 | 0 | 11.76 | 58.82 |

To clarify the behavior of the bedload formulas at different cross sections, having varied morphological characteristics, the formulas were applied for a range of discharges between 400 and 700 m3s-1 at some sections along the reach.

ure (8) show that Einstein’s formula was over-predicting in all sections and it showed multiple points of change in the slope at cross sections CS1, CS6-1, CS7 and CS9 depending on the water flow, whilst at sections CS6-4, CS11 and CS14, the formula curves were smoother. The Meyer-Peter1934 and Schoklitsch1943 formulas were always under-predicting. The Schoklitsch1934 and van Rijn formulas fluctuated between the measurements being under and over depending on the characteristics of the cross section.

It is not clear that there is a unique prediction formula that can predict the bedload discharge with stable magnitude of error along the whole study reach. The important conclusion from the application results is, even for those formulas have agreement with field measurements in regular cross sections, there is no grantee to have the same agreement in the irregular cross sections (meanders, sand bars, etc.).

Annual bed load quantities were computed using all the formulas for the period 2009-2013 along the study reach and are listed in table 3. The annual bedload quantities are ranging from 36 thousand ton (minimum) in 2009 to 50 thousand ton (maximum) in 2013 according to van Rijn1984 formula.

Table 3 Annual bedload predicted discharges for the period 2009~2013.

| Year | 2009 | 2010 | 2011 | 2012 | 2013 | Average |
| --- | --- | --- | --- | --- | --- | --- |
| Meyer-Peter 1934 | 17.88 | 21.07 | 19.59 | 21.96 | 24.61 | 21.02 |
| Schoklitsch 1934 | 50.51 | 64.89 | 57.52 | 68.51 | 81.87 | 64.66 |
| DuBoy 1935 | 1727.07 | 2009.06 | 1880.12 | 2087.37 | 2318.04 | 2004.33 |
| Casey 1935 | 680.44 | 787.01 | 739.41 | 818.28 | 902.62 | 785.55 |
| Shield 1936 | 4085.10 | 4919.11 | 4512.47 | 5125.81 | 5885.69 | 4905.63 |
| Schoklitsch 1943 | 17.52 | 20.01 | 18.93 | 20.75 | 22.66 | 19.97 |
| Kalinske 1947 | 1116.36 | 1187.90 | 1170.03 | 1215.75 | 1246.16 | 1187.24 |
| M-P-M 1948 | 809.65 | 908.84 | 868.22 | 939.89 | 1010.68 | 907.46 |
| Einstein 1950 | 163.65 | 178.58 | 173.43 | 183.72 | 193.62 | 178.60 |
| Enistein-Brown 1950 | 572.10 | 649.50 | 616.26 | 672.13 | 728.98 | 647.79 |
| Rottner 1959 | 548.02 | 635.31 | 595.40 | 657.92 | 735.67 | 634.46 |
| Yalin 1963 | 2330.52 | 2636.03 | 2507.32 | 2729.70 | 2955.13 | 2631.74 |
| Chang et al. 1965 | 282.77 | 337.27 | 309.33 | 344.63 | 409.49 | 336.70 |
| Bagnold 1966 | 897.41 | 1019.74 | 967.51 | 1056.16 | 1147.13 | 1017.59 |
| van Rijn 1984 | 36.17 | 42.58 | 39.56 | 44.23 | 50.50 | 42.61 |
| Wilcock 2001 | 461.07 | 527.38 | 498.35 | 546.79 | 598.19 | 526.36 |
| Julien 2002 | 1441.44 | 1644.19 | 1555.99 | 1703.37 | 1860.09 | 1641.02 |
| Wilcock-Crowe 2003 | 576.33 | 659.22 | 622.94 | 683.49 | 747.74 | 657.95 |
| Camenen-Larson 2005 | 1024.46 | 1158.42 | 1102.02 | 1199.45 | 1298.11 | 1156.49 |
| Wong-Parker 2006 | 399.76 | 448.85 | 428.72 | 464.20 | 499.27 | 448.16 |

Fig. 8 Application of bedload formulas and rating curve at different cross sections for a range of discharges.

- Conclusions

The implemented regulation scheme on the Tigris River has limited the sources of sediment supply; it has also decreased the average water flow to 44% compared to previous periods.

The spatial distribution of the bedload was effected by the bed shear and the flow velocity at the sampling point. Whenever the velocity and the bed shear increase, the bedload increases also for certain limit depending on the particle size then it may transfer to suspension. Some exceptions are expected in the meanders due to the secondary current, where the bedload increases for a lower velocity and/or bed shear.

The complicated geometry of the river reach makes finding a unique representative bedload formula along the study reach rather difficult. Furthermore, even for those formulas having agreement with field measurements in regular cross sections, there is no grantee to have the same agreement in the irregular cross sections (meanders, sand bars, etc.).

The closest bedload prediction formulas were van Rijn1984 then Schoklitsch1934 and the average discrepancy ratios were of the order 1.18 and 1.51 respectively.

Annual bedload quantities were estimated for the period 2009-2013 to be 36 thousand tons (minimum) in 2009 and ranged to 50 thousand tons (maximum) in 2013 according to the van Rijn1984 formula. The average annual transport rate for the period 2009-13 was 42.6 thousand tons.

- References

Al-Ansari, N.A., 2013. Management of Water Resources in Iraq: Perspectives and Prognoses, Engineering, 5(8), 667-684.

Al-Ansari, N.A., 2016. Hydropolitics of the Tigris and Euphrates Basins, Engineering, 8(3), 140-172.

Al-Ansari, N.A., Ali, A. and Knutsson, S., 2014. Present conditions and Future Challenges of Water Resources Problems in Iraq. J. Water Resources and Protection, 6 (12), 1066-1098.

Al-Ansari, N.A., Ali, A., Al-suhail, Q. and Knutsson, S., 2015b. Flow of River Tigris and its Effect on the Bed Sediment within Baghdad, Iraq. Open Engineering, 5, 465-477.

Al-Ansari, N.A., Ali, A., and Knutsson, S., 2015a. Iraq Water Resources Planning: Perspectives and Prognoses. ICCCE 2015: XIII International Conference on Civil and Construction Engineering. Jeddah, 26-27 January, 2097-2108.

Al-Ansari, N.A., and Toma, A., 1984. Bed characteristics of the River Tigris within Baghdad. J. Water Res., 3(1), 1-23.

Ali, A.A., 2013. Morphology of Tigris River inside Baghdad City. Thesis (Licentiate). Lulea University of Technology.

Ali, A.A., Al-Ansari, N.A. and Knutsson, S., 2012. Morphology of Tigris River within Baghdad City. Journal of Hydrology and Earth System Sciences, 16, 3783-3790.

Ali, A.A., Al-Suhail, Q., Al-Ansari, N.A. and Knutsson, S., 2014. Evaluation of Dredging Operations for Tigris River within Baghdad, Iraq. J. Water Resources and Protection, 6 (4), 202-213.

Al-Shahrabaly, Q.M., 2008. River discharges for Tigris and Euphrates gauging stations (in Arabic). Baghdad: Ministry of Water Resources.

Andrews, E.D., 1981. Measurement and computation of bed material discharge in a shallow sand-bed stream, Muddy Creek, Wyoming. Water Recourses Research, 17(1), 131-141.

Atkinson, E., 1994. Vortex-tube sediment extractors I: trapping efficiency. J. Hyd. Engr. ASCE, 120(10), 1110–25.

Beschta, R.L., 1981. Increased bag-size improves Helley-Smith bedload sampler for use in streams with high sand and organic matter transport. Int. Assoc. of Hydro. Sci., a Syrup. 133, 17-25.

Bridge, J. S., and Best, J. L., 1988, Flow, sediment transport and bedform dynamics over the transition from dunes to upper-stage plane beds: implication for the formation of planar laminae. Sedimentology, 35, 753-763.

Bridge, J. S., and Jarvis, J., 1982. The dynamics of a river bend: a study in flow and sedimentary processes. Sedimentology. 29, 499-541.

Bunte, K and Abt, S.R., 2009. Transport Relationships between Bedload Traps and a 3-Inch Helley-Smith Sampler in Coarse Gravel-Bed Streams and Development of Adjustment Functions. Completion Report No. 218. Colorado Water Institute.

Camenen, B. and Larson, M., 2005. A general formula for non-cohesive bed load sediment transport. Estuarine, Coastal and Shelf Science, 63, 249–260.

Cook, K.L., Turowski, J.M, and Hovius, N., 2013. A demonstration of the importance of bedload transport for fluvial bedrock erosion and knickpoint propagation. Earth Surf. Process. Landforms, 38, 683–695.

Dietrich, W.E., and Smith, J.D., 1984. Bedload transport in a river meander. Water Resour. Res., 20, 1355-1380.

Diplas, P., Kuhnle, R., Gary, J., Glysson, D., and Edwards, T., 2008. Sediment transport measurements. In: Garcia, M., ed. Sedimentation Engineering: Processes, Measurements, Modeling, and Practice. Virginia: ASCE manuals and reports on engineering practice No. 110, American Society of Civil Engineers.

Druffel, L., Emmett, W.W., Schneider, V.R., and Skinner, J.V., 1976. LABORATORY HYDRAULIC CALIBRATION OF THE HELLEY- SMITH BEDLOAD SEDIMENT SAMPLER. Mississippi: DEPARTMENT OF THE INTERIOR-GEOLOGICAL SURVEY.

Edwards, T.K. and Glysson, G.D., 1998. Field Methods for Measurement of Fluvial Sediment. In: Applications of Hydraulics. Techniques of Water-Resources Investigations of the U.S. Geological Survey, Denver.

Emmett, W.W. and Hubbell, D.W., 1977. Hydraulic Efficiency and Sampling Efficiency of Helley-Smith Bedload Sampler.

Emmett, W.W., 1980. A Field Calibration of the Sediment-Trapping Characteristics of the Helley-Smith Bedload Sampler. GEOLOGICAL SURVEY PROFESSIONAL PAPER 1139.

Friend, P.F., 1993. Control of river morphology by the grain-size of sediment supplied. Sediment. Geol., 85, 171-177.

Gaudet, J.M., Roy, A.G. and Best, J.L., 1994. EFFECT OF ORIENTATION AND SIZE OF HELLEY-SMITH SAMPLER ON ITS EFFICIENCY. J. Hydraul. Eng., 120, 758-766.

Gaweesh, M.T.K. and van Rijn, L.C., 1994. BED-LOAD SAMPLING IN SAND-BED RIVERS. Journal of Hydraulic Engineering, 120 (12), 1364-1384.

Geohydraulique, 1977. Tigris River Training Project within Baghdad City. Report Submitted to the Iraqi Ministry of Irrigation, Paris.

Graf, W.H., 1971. Hydraulics of sediment transport. USA: McGraw-Hill.

Hardy, F.S., Lacey, G., ALLEN, F.H., WOLF, P.O., ADDISON, H., RICHARDS, B.D., STEPHENSON, H.W., DATH, F.P., and MIDDLETON, A.A., 1956. Discussion on SOME HYDRAULIC INVESTIGATIONS IN CONNEXION WITH THE WADI THARTHAR PROJECT, IRAQ. ICE proceeding: engineering divisions. 5 (4), 352-365.

Helley, E.J. and Smith, W., 1971. DEVELOPMENT AND CALIBRATION OF A PRESSURE-DIFFERENCE BEDLOAD SAMPLER. OPEN-FILE REPORT, USGS, California.

Hickin, E.J., 1995. River Geomorphology. Wiley.

Julien, P.Y., 2002. River Mechanics. USA: Cambridge University Press.

Julien, P.Y., 2010. Erosion and Sedimentation. USA: Cambridge University Press.

Khalaf, R.M., 1988. Modelling sediment transport load in Tigris River within Baghdad City. Thesis (M.Sc.), University of Baghdad, Iraq.

Ministry of Water Resources - Iraq (MWR), 2005. Schematic diagram of main control structures in Iraq. General directorate of water resources management, hydrological studies centre.

Recking, A., 2009. Theoretical development on the effects of changing flow hydraulics on incipient bed load motion. WATER RESOURCES RESEARCH, 45 (W04401).

Shen, H.W., 1971. River Mechanics. Colorado: Shen.

Strahlhofer, L.K., 2010. Continuous Bedload Transport Measurements and Calibration Purposes using a Hydraulic Liftable Slot Trap. Thesis (M.Sc.). Lincoln University.

Talukdar, S., Kumar, B. and DUTTA, S., 2012. PREDICTIVE CAPABILITY OF BEDLOAD EQUATIONS USING FLUME DATA. J. Hydrol. Hydromech., 60 (1), 45–56.

University of Technology - Baghdad, 1992. Training of Tigris River inside Baghdad City. Report Submitted to the Iraqi Ministry of Agriculture and Irrigation, Baghdad.

University of Victoria, 2010. Dams in the Tigris Euphrates river basins [Online], http://hdl.handle.net/1828/2400. Online accessed 8th Dec. 2014.

van Rijn, L.C., 1993. PRINCIPLES OF SEDIMENT TRANSPORT IN RIVERS, ESTUARIES AND COASTAL SEAS. Amsterdam: Aqua Publications.

van Rijn, L.C., 2007. Manual sediment transport measurements in rivers, estuaries and coastal seas. Amsterdam: Aqua publications.

van Rijn, L.C., and Gaweesh, M.T.K., 1992. NEW TOTAL SEDIMENT-LOAD SAMPLER. Journal of Hydraulic Engineering, 118, 1686-1691

Vanoni, V.A., 2006. Sedimentation Engineering. ASCE manuals and reports on engineering practice No. 54. Virginia: ASCE.

Walling, D.E., 1977. Assessing the Accuracy of Suspended Sediment Rating Curves for a Small Basin. Water Resources Research, 13(3), 531-538.

Wilcock, P.R. and Crowe, J.C., 2003. Surface-based Transport Model for Mixed-Size Sediment. Journal of Hydraulic Engineering, 129 (2), 120-128.

Wilcock, P.R., 2001. TOWARD A PRACTICAL METHOD FOR ESTIMATING SEDIMENT-TRANSPORT RATES IN GRAVEL-BED RIVERS. Earth Surface Processes and Landforms, 26, 1395–1408

Wong, M. and Parker, G., 2006. RE-ANALYSIS AND CORRECTION OF BEDLOAD RELATION OF MEYER-PETER AND MÜLLER USING THEIR OWN DATABASE. Journal of Hydraulic Engineering, 132 (11), 1159-1168.

World Meteorological Organization (WMO), 2003. MANUAL ON SEDIMENT MANAGEMENT AND MEASUREMENT. OPERATIONAL HYDROLOGY REPORT No. 47, WMO-No. 948.

Yalin, S.M., 1977. Mechanics of Sediment Transport. UK: Pergamon Press.

Yang, C.T., 1996. Sediment transport: theory and practice. USA: McGraw-Hill.

Figures captions:

Fig. 1 (A) Map of Iraqi Provences (B) Tigris River inside Baghdad, capital of Iraq (islands and sandbars bordered by red) (Ali et al. 2014, 2015)

Fig. 2 Schematic Diagram of Tigris River Hydrological Scheme (MWR 2005).

Fig. 3 Decadal hydrographs of Tigris River at Sarai Baghdad for the period 1930-2013 (data source: Al-Shahrabaly 2008).

Fig. 4 Contribution of runoff of Adhaim River sub-basin downstream Adhaim Dam to Tigris River discharges for the year 2004~2005.

Fig. 5 Bedload sampling cross sections along the northern part of Tigris River (Ali et al. 2014, 2015)

Fig. 6 The measurements of the bedload discharges with the measured velocity and the calculated bed shear stresses at the sampling points along the Tigris River study reach.

Fig. 7 Comparison of bedload discharges predicted by different bedload formulas with measured bedload in Tigris River.

Fig. 8 Application of bedload formulas and rating curve at different cross sections for a range of discharges.